# RoboCupJunior Soccer 2026

Field & Ball Specifications — Summary

This document summarizes the official 2026 RoboCupJunior Soccer field and ball specifications (Final rules as of June 3, 2026), for quick reference while building or practicing on a field.

# Field Dimensions

| Element | Measurement |
| --- | --- |
| Playing field | 158 cm × 219 cm |
| Outer area (beyond white boundary line) | 12 cm wide, all sides |
| Total field footprint (incl. outer area) | 182 cm × 243 cm |
| Wall height | 22 cm, painted matte black |
| Recommended field height off the ground | 70–90 cm |

# Goals

- Inner space: 60 cm wide, 10 cm high, 74 mm deep, box-shaped.

- Goal posts: Positioned directly over the white boundary line.

- Interior color: Matte-colored — one goal yellow, the other blue. Blue is recommended to be a brighter shade so it stays visually distinct from the black walls.

# Floor & Markings

- Surface: Green carpet, ideally a darker shade, over a hard level surface. Shade can vary between venues.

- Line width: 20mm wide (±10%), used for the field boundary and penalty areas.

- Center circle: 60 cm diameter, thin black marker line, centered on the field.

- Neutral spots: Five total — one at field center, four located 45 cm from each short edge (aligned with the penalty area sides). Each is a 1 cm diameter circle.

- Penalty areas: 25 cm wide, 80 cm long, with 15 cm-radius rounded front corners, marked by a 20mm white line.

# Lighting & Magnetic Conditions

Organizers aim to limit external light and magnetic interference, but robots must be built to tolerate imperfect conditions — don't rely solely on compass sensors or assume ideal, consistent lighting.

# Ball Specification

| Property | Value |
| --- | --- |
| Diameter | 42mm ± 1mm |
| Mass | 46 ± 3 grams |
| Color | Orange (standard golf ball) |

Note on Regional Variation These are the international/master ruleset dimensions. Regional and local competitions may adapt field specs — confirm with your tournament organizer if practicing on a self-built or borrowed field.